Here is a GSM Library.This Library is derived from Hwkitchen’s GSM Library and include the NewSoftSerial library to comunicate using the pin 10 (RX) and 8 (TX) of raspberry pi.

# How install the library for raspberry pi

After downloading of the GSM Library unzip the GSM folder to raspberry pi Desktop. In the GSM.zip you can find the library to comunicate with gsm module and the file for use the NewSoftSerial. Input “make install” to install this library int this file.

# Basic description

GSM library is created as standard class with the GSM file source files. GSM class is based mainly on the serial communication between the Raspberry pi board and the GSM module placed on the GSM Shield. There are used standard AT commands for the communication with the GSM Module. The Current version of library uses blocking version of communication. It means that the program is blocked until the communication function is finished - so until required data are sent and required answer is received. The Advantage of that blocking approach is that it is easy to understand the program flow. On the other hand there is also disadvantage that we can’t use the processor resources in the time when the program just waits for the incoming data.

### Methods

There are described important functions for the end user in this document. The GSM Library contains also some functions that are used internally and are not described in the document. It is also possible to use these functions by the end user of course as they are defined as public but there is needed to check the library source code with notes.

int LibVer(void):

returns library version in format XYY -it means X.YY (e.g. 100 means vers. 1.00)

void serialBegin(int baud)

open the serial in user baud to communicate with the GSM module.

void TurnOn(baud):

turns on the GSM module in case module is off and sends some initialization AT commands which are possible to send before registration ->

void InitParam(byte group):

Sends parameters for initialization of GSM module

group:

0 - parameters of group 0 - not necessary to be registered in the GSM AT&F

1 -parameters of group 1 - it is necessary to be registered

AT+CLIP=1

AT+CMEE=0

AT+CMGF=1

void Echo(byte state):

Function to enable or disable echo

Echo(1)   enable GSM echo mode

Echo(0)   disable GSM echo mode

byte CheckRegistration(void):

checks if the GSM module is registered in the GSM net. This method communicates directly with the GSM module in contrast to the method IsRegistered() which reads the flag from the module_status (this flag is set inside this method) must be called regularly at the one place in the main sketch loop function (recommendation repeat time is from 1sec. to 10 sec.)

return valus:

REG_NOT_REGISTERED - not registered

REG_REGISTERED - GSM module is registered

REG_NO_RESPONSE - GSM doesn’t response

REG_COMM_LINE_BUSY - comm line is not free

byte IsRegistered(void):

returns flag if the GSM module is registered in the GSM net this method does not communicate directly with the GSM module,just reads the flag so it is very fast unlike CheckRegistration() which takes more than 20msec. it is recommended to use this function everytime it is necessary to use some GSM function which needs GSM module is registered – checking ingoing SMSs, checking calls etc.

return valus:

0 - not registered

>0 - GSM module is registered

byte CallStatus(void):

checks call status

return values:

CALL_NONE - no call

CALL_INCOM_VOICE - incoming voice call

CALL_ACTIVE_VOICE - active voice call

CALL_NO_RESPONSE - no response

byte CallStatusWithAuth(char *phone_number, byte first_authorized_pos, byte last_authorized_pos):

checks status of call(incoming or active) and makes authorization with specified SIM positions range

parameters and return values:

phone_number: a pointer where the tel. number string of current call will be placed so the space for the phone number string must be reserved

first_authorized_pos: initial SIM phonebook position where the authorization process starts

last_authorized_pos: last SIM phonebook position where the authorization process finishes

Note(important):

In case first_authorized_pos=0 and also last_authorized_pos=0 the received incoming phone number is NOT authorized at all, so every incoming is considered as authorized (CALL_INCOM_VOICE_NOT_AUTH is returned)

return:

CALL_NONE - no call activity

CALL_INCOM_VOICE_AUTH - incoming voice - authorized

CALL_INCOM_VOICE_NOT_AUTH - incoming voice - not authorized

CALL_ACTIVE_VOICE - active voice

CALL_INCOM_DATA_AUTH - incoming data call - authorized

CALL_INCOM_DATA_NOT_AUTH - incoming data call - not authorized

CALL_ACTIVE_DATA - active data call

CALL_NO_RESPONSE - no response to the AT command

CALL_COMM_LINE_BUSY - comm line is not free

void PickUp(void):

picks up the incoming call

void HangUp(void):

hangs up call(incoming or active)

void Call(char *number_string):

calls the specific number

e.g. Call(“+390123456789”);

void Call(int sim_position):

calls the number stored at the specified SIM position

e.g. Call(1); // call to the number stored at the 1st SIM position

char SendSMS(char *number_str, char *message_str):

sends SMS to the specific phone number

parameters and return values:

number_str: pointer to the phone number string

message_str: pointer to the SMS text string

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - GSM module has answered “ERROR” string

- OK ret val:

0 -SMS was not sent

1 -SMS was sent 9/15

example of use:

SendSMS(“00XXXYYYYYYYYY”, “SMS text”);

char SendSMS(byte sim_phonebook_position, char *message_str)

sends SMS to the specified SIM phonebook position

parameters and return values:

sim_phonebook_position: SIM phonebook position <1..20>

message_str: pointer to the SMS text string

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - specified position must be > 0

- OK ret val:

0-SMS was not sent

1-SMS was sent

an example of usage:

GSM gsm;

SendSMS(1, “SMS text”);

char IsSMSPresent(byte required_status):

finds out if there is present at least one SMS with specified status if there is new SMS before IsSMSPresent() is executed this SMS has a status UNREAD and then after calling IsSMSPresent() method status of SMS is automatically changed to READ

parameters and return values:

required_status:

SMS_UNREAD - new SMS - not read yet

SMS_READ - already read SMS

SMS_ALL - all stored SMS

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- OK ret val:

0 - there is no SMS with specified status

1..20 - position where SMS is stored

example of use:

char position;

char phone_number[20]; // array for the phone number string

char *sms_text;

position = IsSMSPresent(SMS_UNREAD);

if (position)

{ // read new SMS

GetGSM(position, tel_number, &sms_text);

}

char GetSMS(byte position, char *phone_number, char *SMS_text, byte max_SMS_len):

reads SMS from specified memory(SIM) position

parameters and return values:

position: SMS position <1..20>

phone_number: a pointer where the phone number string of received SMS will be placed so the space for the phone number string must be reserved - see example

SMS_text : a pointer where SMS text will be placed

max_SMS_len: maximum length of SMS text excluding also string terminating 0x00 character

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - specified position must be > 0

- OK ret val:

GETSMS_NO_SMS - no SMS was found at the specified position

GETSMS_UNREAD_SMS - new SMS was found at the specified position

GETSMS_READ_SMS - already read SMS was found at the specified position

GETSMS_OTHER_SMS - other type of SMS was found an example of usage:

GSM gsm;

char position;

char phone_num[20]; // array for the phone number string

char sms_text[100]; // array for the SMS text string

position = IsSMSPresent(SMS_UNREAD);

if (position)

{

// there is new SMS => read it

GetGSM(position, phone_num, sms_text, 100);

Printf(“DEBUG SMS phone number: “, 0);

Printf(phone_num, 0);

Printf(“\r\n SMS text: “, 0);

Printf(sms_text, 1);

}

char GetAuthorizedSMS( byte position, char *phone_number, char *SMS_text, byte max_SMS_len, byte first_authorized_pos, byte last_authorized_pos):

reads SMS from specified memory(SIM) position and makes authorization - it means SMS phone number is compared with specified SIM phonebook position(s) and in case numbers match GETSMS_AUTH_SMS is returned, otherwise GETSMS_NOT_AUTH_SMS is returned

parameters and return values:

position: SMS position to be read <1..20>

phone_number: a pointer where the tel. number string of received SMS will be placed so the space for the phone number string must be reserved - see example

SMS_text : a pointer where SMS text will be placed

max_SMS_len: maximum length of SMS text excluding terminating 0x00 character

first_authorized_pos: initial SIM phonebook position where the authorization process starts

last_authorized_pos: last SIM phonebook position where the authorization proces finishes

Note(important):

In case first_authorized_pos=0 and also last_authorized_pos=0

the received SMS phone number is NOT authorized at all, so every

SMS is considered as authorized (GETSMS_AUTH_SMS is returned)

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - position must be > 0

- OK ret val:

GETSMS_NO_SMS - no SMS was found at the specified position

GETSMS_NOT_AUTH_SMS - NOT authorized SMS found at the specified position

GETSMS_AUTH_SMS - authorized SMS found at the specified position

an example of usage:

GSM gsm;

char phone_num[20]; // array for the phone number string 12/15

char sms_text[100]; // array for the SMS text string

// authorize SMS with SIM phonebook positions 1..3

if (GETSMS_AUTH_SMS == GetAuthorizedSMS(1, phone_num, sms_text, 100, 1, 3))

{

// new authorized SMS was detected at the SMS position 1

Printf(“DEBUG SMS phone number: “, 0);

Printf(phone_num, 0);

Printf(“\r\n SMS text: “, 0);

Printf(sms_text, 1);

}

// don’t authorize SMS with SIM phonebook at all

if (GETSMS_AUTH_SMS == GetAuthorizedSMS(1, phone_num, sms_text, 100, 0, 0))

{

// new SMS was detected at the SMS position 1

// because authorization was not required

// SMS is considered authorized

Printf(“DEBUG SMS phone number: “, 0);

Printf(phone_num, 0);

Printf(“\r\n SMS text: “, 0);

Printf(sms_text, 1);

}

char DeleteSMS(byte position)

deletes SMS from specified SMS position

parameters and return values:

position: SMS position <1..20>

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - position must be > 0

- OK ret val:

0-SMS was not deleted

1-SMS was deleted

char GetPhoneNumber(byte position, char *phone_number)

reads phone number string from specified SIM position

parameters and return values:

position: SMS position <1..20>

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - position must be > 0

phone_number is empty string

OK ret val:

0-there is no phone number on the position

1-phone number was found

phone_number is filled by the phone number string finished by 0x00

so it is necessary to define string with at least

15 bytes(including also 0x00 termination character)

an example of usage:

GSM gsm;

char phone_num[20]; // array for the phone number string

if (1 == GetPhoneNumber(1, phone_num))

{

// valid phone number on SIM pos. #1

// phone number string is copied to the phone_num array

Printf(“DEBUG phone number: “, 0);

Printf(phone_num, 1);

}

else

{

// there is not valid phone number on the SIM pos.#1

Printf(“DEBUG there is no phone number”, 1);

}

char WritePhoneNumber(byte position, char *phone_number)

writes phone number string to the specified SIM position

parameters and return values:

position: SMS position <1..20>

phone_number: phone number string for the writing

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - position must be > 0

- OK ret val:

0-phone number was not written

1-phone number was written 14/15

char DelPhoneNumber(byte position)

del phone number from the specified SIM position

parameters and return values:

position: SIM position <1..20>

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - position must be > 0

- OK ret val:

0-phone number was not deleted

1-phone number was deleted

char ComparePhoneNumber(byte position, char *phone_number)

compares specified phone number string with phone number stored at the specified SIM position

parameters and return values:

position: SMS position <1..20>

phone_number: phone number string which should be compare

return:

ERROR ret. val:

- 1 - comm. line to the GSM module is not free

- 2 - GSM module didn’t answer in timeout

- 3 - position must be > 0

- OK ret val:

0-phone numbers are different

1-phone numbers are the same

an example of usage:

if (1 == ComparePhoneNumber(1, “123456789”))

{

// the phone num. “123456789” is stored on the SIM pos. #1

// phone number string is copied to the phone_num array

Printf(“DEBUG phone numbers are the same”, 1);

}

else

{

Printf(“DEBUG phone numbers are different”, 1);

}